SecureGate

An IoT security project for granting controlled access to infrastructure using multi factor authentication.

| DOCUMENT INFORMATION |  |  |
| --- | --- | --- |
| Student Name: |  | Kieron Garvey |
| Student#: |  | 96358157 |
| eMail: |  | 96358157@mail.wit.ie |
| Supervisor: |  | Frank Walsh |
| Module: |  | Placement & Project |
| Course: |  | Higher Diploma in Computer Science |
| Version: |  | 1.0 |
| Effective Date: |  | 23rd January 2025 |

| Declaration of Authenticity |  |  |  |
| --- | --- | --- | --- |
| I declare that the work which follows is my own, and that any quotations from any sources (e.g. books, journals, the internet) are clearly identified as such by the use of ‘single quotation marks’, for shorter excerpt and identified italics for longer quotations. All quotations and paraphrases are accompanied by (date, author) in the text and a fuller citation is the bibliography. I have not submitted the work represented in this report in any other course of study leading to an academic award. |  |  |  |
| Student: | Kieron Garvey______ | Date: | 23/01/2025______ |

Acknowledgements

TBD

Preface

TBD

| Revision History |  |  |  |
| --- | --- | --- | --- |
| Revision # | Description | Author | Effective Date |
| 1.0 | Initial Draft Submission | Kieron Garvey | 23/01/2025 |

Table of Contents

1	Introduction	4

1.1	Objective	4

1.2	Purpose	4

1.3	AMS (Original) Proposal	4

1.4	Acronyms and Abbreviations	5

2	Research and Analysis	5

2.1	Context	5

2.2	Existing Solutions	5

2.3	Tools	6

2.3.1	Proxmox	6

2.3.2	Portainer	6

2.3.3	Docker	6

2.3.4	TinyTuya	6

2.3.5	Supabase	6

2.3.6	MQTT (Message Queuing Telemetry Transport)	7

2.3.7	OpenCV	7

2.3.8	Face-Recognition	7

2.3.9	Fast API	7

2.4	Technologies	8

2.4.1	Python	8

2.4.2	React	8

2.4.3	React Native	8

2.4.4	Flutter	8

2.5	Equipment	8

Rekavin USB Plug	11

3	Design & Modeling	12

3.1	User Persona	12

3.1.1	Admin	12

3.1.2	Chemist	13

3.1.3	LAB TECHNICIAN (Senior)	14

3.1.4	LAB TECHNICIAN (Junior)	15

3.1.5	Cleaner	16

3.2	User Interface Design	17

3.2.1	User Case Diagram	17

3.2.2	Front-End Interface Design	17

3.2.3	Back-End Interface Design	17

3.3	Database Schema	19

3.4	Physical Model Design	24

3.4.1	Electrical Drawing	24

3.4.2	Front View Design	25

3.4.3	Side View Design	25

4	Conclusion + Further work	26

4.1	Reflection	26

4.1.1	What I have learned	26

4.1.2	What I would do differently	26

5	References	26

6	Attachments	26

6.1	Attachment#1:- Electrical Drawing	27

6.2	Attachment#2:-	28

6.3	Attachment#3:-	28

# Introduction

## Objective

Automated Management Systems (AMS) was a project I created for the Computer Systems & Networks Module of the Higher Diploma in Computer Science course.

My objective is to refactor this project removing several of its dependencies, rename it too SecureGate and making it more business friendly.

## Purpose

## AMS (Original) Proposal

AMS is an innovative IoT project focused on enhancing security and privacy in a controlled company environment. AMS addressed the challenges of granting controlled access to a room and the devices within the room without compromising sensitive information.

AMS integrates facial recognition and remote access technologies to regulate entry, ensuring only authorized personnel enter the controlled environment. It employs sophisticated security monitoring to restrict access to specific devices, preventing unauthorized usage within the environment.

The system optimizes energy consumption by selectively powering on devices based on user permissions. When multiple individuals are present, AMS requires facial recognition or QR code authentication for device activation, ensuring access control.

Additionally, AMS implements secure login protocols for devices, requiring facial recognition and/or QR code authentication, thereby enhancing security measures within the controlled environment. By providing detailed access control and authentication mechanisms, AMS ensures heightened security and restricted access while optimizing resource utilization.

| Original Project Documents |  |
| --- | --- |
| DESCRIPTION | URL |
| AMS Proposal | https://github.com/ki321g/AMS/blob/main/Documentation/AssignmentProposal.pdf |
| GitHUB Repository | https://github.com/ki321g/AMS |
| Video Walk Through | https://youtu.be/SJLO5kULhWg |

## Acronyms and Abbreviations

| Name | Description |
| --- | --- |
| AMS | Automated Management Systems |
| VM | Virtual Machine |
| DC | Docker Container |
| API | Application Programming Interface |
| WiFi | Wireless Fidelity |
| LAN | Local Area Network |
| MQTT | Message Queuing Telemetry Transport |
| OpenCV | Open-Source Computer Vision Library |
| dlib's | dlib: A toolkit for machine learning and computer vision (not an acronym). |
| UI | User Interface |
| iOS | iPhone Operating System |
| DOM | Document Object Model |
| pHAT | Python Hardware Attachment on Top |
| USB | Universal Serial Bus |
| TPU | Tensor Processing Unit |
| NOIR | No Infrared |
| UML | Unified Modeling Language |
| UUID | Universally Unique Identifier |

# Research and Analysis

planning

## Context

planning

## Existing Solutions

planning

## Tools

I am planning on using some of the following tools this may change as I research the project a bit more.

### Proxmox

Proxmox delivers powerful, enterprise-grade solutions with full access to all functionality for everyone - highly reliable and secure. The software-defined and open platforms are easy to deploy and manage. Proxmox Virtual Environment is a complete open-source platform for enterprise virtualization. With the built-in web interface, you can easily manage VMs and containers, software-defined storage and networking, high-availability clustering, and multiple out-of-the-box tools using a single solution.

URL: https://www.proxmox.com/en/

### Portainer

Portainer is a universal container management platform. It's multi-cluster and multi-device support which means you can manage environments of any type, anywhere (Docker and Kubernetes, running on dev laptops, in your DC, in the cloud, or at the edge), and we don't require you to run any specific Kubernetes distro.

URL: https://www.portainer.io/

### Docker

Docker is an open-source platform designed to simplify the development, deployment, and management of applications. It uses containerization, a technology that packages an application and its dependencies into a portable and isolated environment known as a container.

### TinyTuya

This python module controls and reads state of Tuya compatible WiFi Smart Devices (Plugs, Switches, Lights, Window Covers, etc.) using the local area network (LAN) or the cloud (TuyaCloud API).

URL: https://github.com/jasonacox/tinytuya

### Supabase

Supabase is an open-source Firebase alternative.

Start your project with a Postgres database, Authentication, instant APIs, Edge Functions, Realtime subscriptions, Storage, and Vector embeddings.

URL: https://supabase.com/

### MQTT (Message Queuing Telemetry Transport)

MQTT (Message Queuing Telemetry Transport) is a lightweight messaging protocol designed for efficient communication between devices in IoT and other applications. It enables secure, reliable data transfer with minimal bandwidth usage, ideal for interconnected systems and remote device management.

URL: https://mqtt.org/

### OpenCV

OpenCV (Open Source Computer Vision Library) is a powerful open-source software library designed for real-time computer vision and image processing applications.

URL: https://opencv.org/

### Face-Recognition

Recognize and manipulate faces from Python or from the command line with the world's simplest face recognition library.

Built using dlib's state-of-the-art face recognition built with deep learning. The model has an accuracy of 99.38% on the Labeled Faces in the Wild benchmark.

URL: https://github.com/ageitgey/face_recognition

### Fast API

FastAPI is a modern, high-performance web framework for building APIs with Python. It is designed to deliver robust, efficient, and user-friendly APIs

URL: https://fastapi.tiangolo.com/

## Technologies

Again, I am planning on using the following technologies, but this may change as I research the project a bit more.

### Python

Python is a high-level, versatile programming language known for its simplicity and readability. It's used across various domains such as web development, data analysis, artificial intelligence, scientific computing, and more. Python's clean syntax and extensive libraries make it beginner-friendly while offering powerful capabilities for building applications, automating tasks, handling data, and implementing algorithms efficiently.

### React

React is a popular JavaScript library for building user interfaces, primarily for web applications. Developed and maintained by Facebook, React enables developers to create reusable UI components that efficiently update and render when data changes.

### React Native

React Native is a framework derived from React that focuses on building mobile applications for iOS and Android using JavaScript. Instead of rendering to the web's DOM, React Native components translate to native widgets, delivering a performance and feel akin to apps developed with native technologies.

### Flutter

Flutter is an open-source UI toolkit created by Google for crafting natively compiled applications for mobile, web, and desktop from a single codebase. Unlike React Native, Flutter uses Dart, a programming language also developed by Google. It boasts its own rendering engine, Skia, allowing for complete control over every pixel rendered on the screen. This ensures highly customizable, visually rich UIs that look and feel consistent across platforms.

## Equipment

The system will incorporate the following devices

| Name | Image |
| --- | --- |
| Raspberry Pi 4 & 5 |  |
| Lrtzcbi Touchscreen Portable Monitor 14 Inch |  |
| pHAT Stack Kit for Raspberry Pi |  |
| Raspberry Pi Camera Module 3 Wide NOIR |  |
| USB Accelerator Edge TPU Coporocessor |  |
| ELEGOO Upgraded Electronics Fun Kit |  |
| Mini Portable USB Night Light |  |
| Woox R4785 smart plug |  |
| # Rekavin USB Plug |  |

# Design & Modeling

## User Persona

In this section you’ll find details of the different User persona’s and the users Flow/Journey.

### Admin

#### Persona

|  |  | Name | Sarah Johnson |
| --- | --- | --- | --- |
|  |  | Age Range | 35 |
|  |  | Role | ADMIN |
| Description | - Sarah is responsible for managing the SecureGate system. - She has full access to the Admin backend to add, delete, and update users, devices, and roles. - She doesn’t have access to the Lab. - She does not have access to any physical devices in the lab.Sarah ensures that the system is running smoothly and that all users have the appropriate permissions. |  |  |
| Key Responsibilities | - Manage user accounts (create, update, delete). - Manage devices and roles. - Monitor access logs and device logs for security and compliance. |  |  |
| Access | - No access to physical devices. - Full access to the backend system for user and device management. |  |  |

#### User Flow/Journey

### Chemist

#### Persona

|  |  | Name | Dr. Michael Power |
| --- | --- | --- | --- |
|  |  | Age Range | 48 |
|  |  | Role | CHEMIST |
| Description | - Dr. Power is a senior chemist who oversees all lab operations. - He requires access to all devices in the lab to conduct experiments and monitor results. - He is highly skilled and trusted with sensitive equipment. |  |  |
| Key Responsibilities | - Conduct experiments using lab devices. - Monitor and analyze data from devices. - Ensure devices are used correctly and maintained properly. |  |  |
| Access | - Access to all devices in the lab. |  |  |

#### User Flow/Journey

### LAB TECHNICIAN (Senior)

#### Persona

|  |  | Name | Jenifer Grace |
| --- | --- | --- | --- |
|  |  | Age Range | 33 |
|  |  | Role | Senior Lab Technician |
| Description | - Emily is an experienced lab technician who assists the chemist and manages day-to-day lab operations. - She has access to most devices but not the most sensitive or high-risk equipment. - She is responsible for training junior technicians and ensuring lab protocols are followed. |  |  |
| Key Responsibilities | - Operate and maintain lab devices. - Train junior technicians. - Assist the chemist with experiments. |  |  |
| Access | - Access to most devices, except for high-risk or sensitive equipment. |  |  |

#### User Flow/Journey

### LAB TECHNICIAN (Junior)

#### Persona

|  |  | Name | James O’Connor |
| --- | --- | --- | --- |
|  |  | Age Range | 24 |
|  |  | Role | Junior Lab Technician |
| Description | - James is a recent graduate who is still learning the ropes of lab operations. - He has limited access to devices and is supervised by senior technicians and the chemist. - He is eager to learn and follows strict protocols. |  |  |
| Key Responsibilities | - Assist senior technicians with device operation. - Follow lab protocols and safety guidelines. - Perform routine tasks under supervision. |  |  |
| Access | - Access to basic and low-risk devices. - Cannot access high-risk or sensitive equipment. |  |  |

#### User Flow/Journey

### Cleaner

#### Persona

|  |  | Name | Maria Walsh |
| --- | --- | --- | --- |
|  |  | Age Range | 51 |
|  |  | Role | CLEANER |
| Description | - Maria is responsible for cleaning and maintaining the lab environment. - She does not have access to any lab devices, as her role does not require it. - She works during off-hours to ensure the lab is clean and safe for the next day. |  |  |
| Key Responsibilities | - Clean lab equipment and workspaces. - Dispose of waste materials safely. - Ensure the lab is tidy and organized. |  |  |
| Access | - No access to any lab devices. - Can only access the lab during designated cleaning hours. |  |  |

#### User Flow/Journey

## User Interface Design

### User Case Diagram

#### Admin

TBD

#### Cleaner

TBD

#### All Other Users

TBD

### Front-End Interface Design

The Front-End Interface designs are the designs of application that everyone would use to unlock and select the device they want to turn on.

#### Idle

TBD

#### Scanning Card

TBD

#### Facial Recognition

TBD

#### Selecting Device

TBD

### Back-End Interface Design

The Back-End Interface designs are the designs of application that the admin will use to ADD, DELETE, UPDATE the users, devices and roles.

#### Users

TBD

#### Devices

TBD

#### Roles

TBD

## Database Schema

Below is a detailed breakdown of the database schema, including relationships between tables and a UML diagram to visualize the database model.

Database Schema Breakdown

Schema Name: securegate

- ROLES Table

Purpose: Stores the roles that users can have (e.g., Admin, Chemist, Lab Technician, Cleaner).

Table Fields:

  - role_id (UUID, Primary Key): Unique identifier for the role.

  - role_name (TEXT, Unique): Name of the role.

  - description (TEXT): Optional description of the role.

  - created_at (TIMESTAMP WITH TIME ZONE): Timestamp when the role was created.

- DEVICES Table

Purpose: Stores information about devices in the lab.

Table Fields:

  - device_id (UUID, Primary Key): Unique identifier for the device.

  - device_name (TEXT): Name of the device.

  - description (TEXT): Optional description of the device.

  - created_at (TIMESTAMP WITH TIME ZONE): Timestamp when the device was added.

- ROLE_TO_DEVICE Table

Purpose: Stores which roles have access to which devices.

Table Fields:

  - device_id: (UUID, Foreign Key): References devices(device_id) to assign a device to the role.

  - Role_id: (UUID, Foreign Key): References roles(role_id) to assign a role to the device.

  - PRIMARY KEY (role_id, device_id) Composite primary key

- USERS Table

Purpose: Stores user information, including their role and access details.

Table Fields:

  - uid (UUID, Primary Key): Unique identifier for the user.

  - first_name (TEXT): User's first name.

  - last_name (TEXT): User's last name.

  - email (TEXT, Unique): User's email address (validated with a regex check).

  - password (TEXT): Hashed password for authentication.

  - phone_number (TEXT): User's phone number.

  - role_id (UUID, Foreign Key): References roles(role_id) to assign a role to the user.

  - card_uid (TEXT, Unique): Unique identifier for the user's access card.

  - user_picture (TEXT): Base64-encoded image for facial recognition.

  - last_seen_at (TIMESTAMP WITH TIME ZONE): Timestamp of the user's last activity.

  - created_at (TIMESTAMP WITH TIME ZONE): Timestamp when the user was created.

- ACCESS_LOGS Table

Purpose: Logs access attempts to the lab.

Table Fields:

  - log_id (UUID, Primary Key): Unique identifier for the log entry.

  - user_id (UUID, Foreign Key): References users(uid) to identify the user.

  - success (BOOLEAN): Indicates whether the access attempt was successful.

  - notes (TEXT): Reason for access failure (if applicable).

  - user_picture (TEXT): Base64-encoded image captured during the access attempt.

  - created_at (TIMESTAMP WITH TIME ZONE): Timestamp of the access attempt.

- DEVICE_LOGS Table

Purpose: Logs actions performed on devices (e.g., turning on/off).

Table Fields:

  - log_id (UUID, Primary Key): Unique identifier for the log entry.

  - device_id (UUID, Foreign Key): References devices(device_id) to identify the device.

  - user_id (UUID, Foreign Key): References users(uid) to identify the user performing the action.

  - action (TEXT): Description of the action performed (e.g., "Turn On", "Turn Off").

  - status (BOOLEAN): Status of the device after the action (e.g., true for "On", false for "Off").

  - notes (TEXT): Additional notes about the action.

  - created_at (TIMESTAMP WITH TIME ZONE): Timestamp of the action.

Relationships Between Tables

- users and roles:

  - Relationship: Many-to-One.

  - Description: Many users can have the same role.

  - Foreign Key: users.role_id references roles.role_id.

- access_logs and users:

  - Relationship: One-to-Many.

  - Description: One user can have multiple access log entries.

  - Foreign Key: access logs.user_id references users.uid.

- access_logs and devices:

  - Relationship: One-to-Many (optional).

  - Description: One device can have multiple access log entries.

  - Foreign Key: access_logs.device_id references devices.device_id.

- device_logs and users:

  - Relationship: One-to-Many.

  - Description: One user can perform multiple actions on devices.

  - Foreign Key: device_logs.user_id references users.uid.

- device_logs and devices:

  - Relationship: One-to-Many.

  - Description: One device can have multiple action logs.

  - Foreign Key: device_logs.device_id references devices.device_id.

- roles and role_to_device:

  - Relationship: Many-to-Many.

  - Description: One role can have access to many devices, and each device can be accessed by multiple roles.

  - Foreign Key: role_id references roles.role_id.

- devices and role_to_device:

  - Relationship: Many-to-Many.

  - Description: One device can be accessed by many roles, and each role can have access to multiple devices.

  - Foreign Key: device_id references devices.device_id

UML Diagram

## Physical Model Design

Below you will find the drawings for the physical design for the project. These are subject to change.

### Electrical Drawing

### Front View Design

### Side View Design

# Conclusion + Further work

## Reflection

### What I have learned

TBD

### What I would do differently

TBD

# References

TBD

# Attachments

## Attachment#1:- Electrical Drawing

## Attachment#2:-

## Attachment#3:-